OPTIMIZATION OF CONTROL UNIT BASED ON PECULIARITIES OF CPLD

Barkalov A.A., Zelenyova I.Y., Kovalyov S.A., Lavrik A.S. (University of Zielona Gora, DonNTU, Zielona Gora, Donetsk, Poland, Ukraine)

The method of hardware reduction is proposed oriented on compositional microprogram control units and CPLD chips. The method is based on a wide fan-in of PAL macrocells allowing using more than one source of microinstruction address. Such approach permits to minimize the number of PAL macrocells used for transformation of microinstruction address. Conditions for this method application and example of its application are given.

1. Introduction

A control unit is one of the very important parts of any digital system [1]. If a control algorithm to be interpreted is a linear one, then it can be implemented using the model of compositional microprogram control unit (CMCU) [2]. The programmable logic devices with programmable array logic (PAL) macrocells are wide used for implementation of logic circuits of control units [3,4]. The high cost of such devices requires optimization of hardware amount in the circuit of CMCU. One of the ways for this task solution is decrease of the number of conjunctive terms in the sum-of-product (SOP) forms of the functions to be Implemented [5,6]. This article propose some approach for this problem solution based on a wide fan-in of PAL macrocells [3,4]. This approach is targeted on CMCU with address transformer [2] and a control algorithm is represented as graph-scheme of algorithm (GSA) [7].

2. Peculiarities of CMCU with address transformer

Let GSA Γ be represented by sets of vertices B and arcs E. Let B=b0,bE∪E1∪E2, where b0 is an initial vertex, bE is a final vertex, E1 is a set of operator vertices, where E1=M,and E2 is a set of conditional vertices. A vertex bq∈E1 contains a microinstruction Ybq⊆Y, where Y=y1,...,yN is a set of data-path microoperations [7]. Each vertex bq∈E2 contains a single element of a set of logical conditions X=x1,...,xL. Let GSA Γ be a linear GSA, that is a GSA with more than 75% of operator vertices.

Let us form a set of operational linear chains (OLC) C=α1,...,αG for GSA Γ, where each OLC αg∈C is a sequence of operator vertices and each pair of its adjacent components corresponds to some arc of the GSA. Each OLC αg∈C has only one output Og and arbitrary number of inputs. Formal definitions of OLC, its input and output can be found in [2]. Each vertex bq∈E1 corresponds to microinstruction MIq kept in a control memory (CM) of CMCU and it has an address Abq. The microinstructions can be addressed using

| R=log2M | (1) |
| --- | --- |

bits, represented by variables Tr∈T=T1,...,TR. Let OLC αg∈C include Fg components and the following condition take place:

| Abgi+1=Abgi+1, | (2) |
| --- | --- |

In (2) bgi is the i-th component of OLC αg∈C, where i=1,...,Fg−1.

If outputs Oi,Oj are connected with an input of the same vertex, then OLC αi,αj∈C are pseudoequivalent OLC (POLC) [2]. Let us construct the partition ΠC=B1,...,BI, of the set C1⊆C on the classes of POLC. Let us point out that αg∈C1 if Og,BE∉E. Let us encode the classes Bi∈ΠC by binary codes KBi with

| R1=log2I | (3) |
| --- | --- |

bits and use the variables τr∈τ=τ1,...,τR1 for the encoding. In this case a GSA Γ can be interpreted using the model of CMCU U1 with address transformer (Fig. 1).

Fig. 1. Structural diagram of CMCU U1

The pulse Start causes loading of the first microinstruction address into a counter CT and set up of fetch flip-flop TF. If Fetch = 1, then microinstructions can be read out the control memory CM. If a current microinstruction does not correspond to OLC output, then a special variable y0 is formed together with microoperations Yq⊆Y. If y0 = 1, then content of CT is incremented according to the addressing mode (2). Otherwise, the block of microinstruction address BMA generates functions

| Φ=Φτ,X | (4) |
| --- | --- |

to load the next microinstruction address into CT. In the same time, the block of address transformer BAT generates functions

| τ=τT. | (5) |
| --- | --- |

If the output of OLC αg∉C1 is reached, then yE=1. It causes cleaning of TF and operation of CMCU U1 is terminated.

Such organization of CMCU permits decrease of the number of terms in functions Φ from H1 till H0, where H1, H0 is the number of terms for equivalents finite state machines (FSM) Moore and Mealy respectively. But block BAT consumes some macrocells or cells of PROM used for implementation of CM. In this article we propose some CMCU U2, where H2 = H0 and block BAT consumes less hardware then its counterpart in U1. Here H2 means the number of terms in functions Φ for CMCU U2.

3. Main idea of proposed method

Let us point out that logic circuits for BMA, CT, TF and BAT are implemented as the parts of CPLD. To implement the CM one should use PROM chips with t outputs, where t = 1, 2, 4, 8, 16 [3, 4]. Let s address the components of OLC αg∈C1 in such manner that condition (2) takes place and maximal possible amount of classes Bi∈ΠC was represented by a single generalized interval of R-dimensional Boolean space. Such addressing needs a special algorithm which should be developed.

Let ΠC=ΠA∪ΠB, where Bi∈ΠA if this class is represented by one interval, and Bi∈ΠB otherwise. The counter CT is a source of the codes for Bi∈ΠA. If condition

| ΠB=0 | (6) |
| --- | --- |

takes place, then block BAT is absent. Otherwise, only output addresses for OLC from classes Bi∈ΠB should be transformed. It is enough

| R2=logIB+1 | (7) |
| --- | --- |

bits for such encoding, where IB=ΠB and 1 is added to take into account the case when Bi∈ΠA. Let us point out that some part of these codes can be implemented using free outputs of PROM. Let us use hot-one encoding of microoperations [2] when the CM word has N+2 bits. In this case the CM can be implemented using

| R0=N+2t | (8) |
| --- | --- |

chips with enough amount of cells (not less than M). Obviously, that R3 outputs of PROM are free, where

| R3=R0∗t−N−2 | (9) |
| --- | --- |

If condition

| R3≥R2, | (10) |
| --- | --- |

takes place, then CM is a source of the codes for Bi∈ΠB and block BAT is absent. Otherwise, we can represent ΠB as ΠE∪ΠD, where IE=ΠE, ID=ΠD. In this case

| IE=2R3−1, | (11) |
| --- | --- |
| R4=log2(ID+1). | (12) |

The value IE is decremented to represent the situation that Bi∈ΠE. The value of ID is incremented to show the case that Bi∈ΠD. Thus, only the outputs of OLC αg∈Bi should be transformed, where Bi∈ΠD. In common case when Πi≠∅i=A,E,D the following CMCU U2 is proposed for interpretation of GSA Γ (Fig. 2).

In CMCU U2, the codes KABi of classes Bi∈ΠA are represented by variables Tr∈T; the codes KEBi of the classes Bi∈ΠE are represented by variables vr∈V, where V=R3; the codes KDBi of the classes Bi∈ΠD are represented by variables zr∈Z, where Z=R4. In CMCU U2, the block BMA implements functions

| Φ=ΦT,Z,V,X, | (13) |
| --- | --- |

and the block BAT implements functions

| Z=ZT. | (14) |
| --- | --- |

Let symbol Ui(Γj) stands for implementation of GSA Гj by CMCU Ui, and symbol QiΓj for the number of macrocells in logic circuit of the block BAT for CMCU Ui(Γj), where i = 1, 2. Let inputs of q-th macrocell of BAT receive Lq logical conditions and let each macrocell have S inputs. Application of the proposed method has sense if condition

| Lq+R+R3+R4≤S, | (15) |
| --- | --- |

takes place, where q = 1, … , Q2Γj.

Fig. 2. Structure diagram of CMCU U2

In this article the method of CMCU U2 synthesis is proposed with the following steps:

- Construction of the sets C, C1 and ΠC for GSA Г.

- Microinstruction addressing.

- Construction of the sets ΠA, ΠE и ΠD.

- Encoding of the classes Bi∈ΠE∪ΠD.

- Construction of the control memory content.

- Construction of transition table for CMCU.

- Construction of the table for address transformer.

- Synthesis of the logic circuit of CMCU.

4. Example of application of proposed method

Let the sets C=α1,...,α9, C1=α1,...,α8 and ΠC=B1,...,B5 are formed for GSA Г1, where α1=b1,b2, α2=b3,...,b6, α3=b7,b8, α4=b5,...,b13, α5=b4,...,b17, α6=b18,...,b21, α7=b22,...,b25, α8=b26,...,b28, α9=b29,...,b31, B1=α1, B2=α2,α3, B3=α4,α5, B4=α6,α7, B5=α8. Thus, I = 5, R1 = 3, τ=τ1,τ2,τ3, M = 31, R = 5.

Let us address the microinstructions using some modification of algorithm from [2]. Now have Ab1=00000,…,Ab25=110000, Ab26=11100,…,Ab28=11110, Ab29=11001,…,Ab31=11011. Let us construct the Karnaugh map marked by the variables  Tr∈T=T1,...,T5 (Fig. 3). This map contains outputs of OLC αg∈C and code space intervals corresponding to the classes Bi∈ΠC.

The sign * in this map stands for the case when a vertex bq∈E1 with address A(bq) is not the output of OLC αg∈C1. The following code intervals can be derived from Fig. 3: class B1 corresponds to interval 0000*, class В2 to 001**, class В3 to 01*** and 10000, В4 to 101** and 11000, class В5 to 111**. Let us point out that α9∉C1 and class B6=α9 is not considered here.

Fig. 3. Karnaugh map for outputs of OLC

The obtained intervals determine the sets ΠA=B1,B2,B5 and ΠB=B3,B4. Let N=13 for GSA Г1 and t=4 for PROM chips in use. In this case we can get R3 = 1 (from (9)) and R2 = 2 (from (7)). Thus, condition (10) is violated and block BAT should be used in CMCU U2(Г1). Let ΠE=B3, then ΠD=B4. Thus, the sets ΠA, ΠE and ΠD are constructed.

Obviously, V=v1,Z=z1, let KEB3=1,KDB4=1. As it was found, KAB1=0000*,KAB2=001**,KAB5=111**.

The content of control memory is constructed in a trivial way [2] and this step is here omitted. Let us point out, that the cells with addresses 10100 and 11000 include the variable v1 = 1.

Let transitions for classes В2, В3, В4 are described by the following system of generalized transition formula [7]:

| B2→x3b9∨x3b26;B3→x1b18∨x1x2b20∨x1x2b26;B4→x5b27∨x5b5. | (16) |
| --- | --- |

The system (16) determines the fragment of transition table with 7 lines (Table 1).

Table 1.

Fragment of transition table for CMCU U2(Г1)

| Bi | KABi | KEBi | KDBi | bq | Abq | Xh | Φh | h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T1T2T3T4T5 | V1 | Z1 |  |  |  |  |  |
| B2 | 001** | 0 | 0 | b9 | 01000 | x3 | D2 | 1 |
|  |  |  |  | b26 | 11100 | x3 | D1D2D3 | 2 |
| B3 | ***** | 1 | 0 | b18 | 10001 | x1 | D1D5 | 3 |
|  |  |  |  | b20 | 10011 | x1x2 | D1D4D5 | 4 |
|  |  |  |  | b26 | 11100 | x1x2 | D1D2D3 | 5 |
| B4 | ***** | 0 | 1 | b27 | 11101 | x5 | D1D2D3D5 | 6 |
|  |  |  |  | b5 | 00100 | x5 | D3 | 7 |

Connection of this table and system (16) is a transparent one. Let us point out that the case v1 = z1 = 0 corresponds to classes Bi∈ΠA. Otherwise, Bi∈ΠE∪ΠD and content of the column KABi is ignored. This table is a base for construction of the system (13). For example, the following parts of SOP can be derived from Table 1:

|  | D1=T1T2T3v1z1x3∨v1z1∨v1z1x5;D2=T1T2T3v1z1∨v1z1x1x2∨v1z1x5. |
| --- | --- |

The table of BAT is constructed for the classes Bi∈ΠD. In our particular case this table includes 2 lines (Table 2).

Table 2

Table of the block BAT for CMCU U2(Г1)

| αg | AOg | Bi | KDBi | Zj | j |
| --- | --- | --- | --- | --- | --- |
| α6 | 10100 | B4 | 1 | z1 | 1 |
| a7 | 1000 |  |  |  | 2 |

This table is the base to construct the system (14). In our case this system represented as the following one:

z1=T1T2T3T4T5∨T1T2T3T4T5.

Synthesis of logic circuit of CMCU U2(Гj) is reduced to implementation of systems (13)-(14) using PAL macrocells and control memory using PROM chips. These problems are well-known [2,6] and they are not discussed in our article.

Let us point out that table of Block BAT for CMCU U1(Г1) includes 8 lines and R1 = 3. Let macrocell PAL has q=3 terms, then block BAT of CMCU U2(Г1) is implemented using Q2(Г1) = 1 macrocells. If the classes Bi∈ΠC of CMCU U1(Г1) is encoded in the following way: К(В1) = 000,…, К(В5) = 100, then Q1(Г1) = 4 (if q = 3). In both cases the block BMA is implemented with the same amount of macrocells.

5. Conclusion

The proposed method targets on decrease in hardware amount (the number of macrocells) in the circuit of address transformer. In this case the number of macrocells in the block of microinstruction address as well as the number of PROM chips in the block of control memory does not change. The method is based on use of three sources of the codes of pseudoequivalent OLC classes. It is possible due to a wide fan-in of industrial PAL macrocells. Let us point out that block BAT can be eliminated if one of the conditions (6) or (10) takes place.

Our experiments show that the number of macrocells in the block BAT is decreased up to 60-70% in comparison with known methods of CMCU design. The total decrease in hardware amount is up to 10% in comparison with CMCU U1(Г1).

References: 1. DeMicheli G. Synthesis and Optimization of Digital Circuits. – McGraw-Hill, 1994. – 636 pp. 2. Баркалов А.А. Синтез устройств управления на программируемых логических устройствах. – Донецк: ДНТУ, 2002. – 262 с. 3. Altera devices. http://www.altera.com/products/devices/common/dev-family_overview.html. 4. Xilinx CPLDs http://www.xilinx.com/products/silicon_solutions/cplds/index.htm. 5. Грушницкий Р.И., Мурсаев А.Х., Угрюмов Е.П. Проектирование систем с использованием микросхем программируемой логики. – СПб: БХВ. – Петербург, 2002. – 608 с. 6. Соловьев В.В. Проектирование цифровых схем на основе программируемых логических  интегральных схем. – М.: Горячая линия-ТЕЛЕКОМ, 2001. – 636 с.

7. Baranov S. Logic Synthesis for Control Automata. – Kluwer Academic Publishers, 1994. – 312 pp.